# Attachment A: Commercial and Industrial Fuel Combustion

This section discusses emissions from industrial and commercial fuel combustion.  These emissions categories, which are listed below, are associated with numerous types of industries and businesses.  Emissions normally would be included with the point source inventory, or would be co-assessed with process-related emissions for a specific industrial or commercial sector.  Other attachments to this memorandum undertake analyses of process-related emissions and also address emissions from fuel combustion (e.g., see Attachment E – Minerals).  Therefore, this attachment only provides a very rough approximation of total emissions from all commercial and industrial fuel combustion sources.  The purpose of this general discussion is to serve as a coarse reality-check of superior, comprehensive emissions estimates or as a stand-in if comprehensive estimates are unavailable.  Specifically, if these rough estimates far exceed comprehensive estimates (say, by 2 orders of magnitude or more) it is recommended that a district or county should investigate more thoroughly whether all important industrial and commercial fuel consumers have been identified and assessed adequately.

Description	EIC Code	CES Number

Co-generation	020-995-0012-0000	74682

Industrial Stationary IC Engines - Diesel	050-040-1200-0000 	82099

Industrial Distillate Oil Combustion	050-995-1220-0000 	66803

Industrial Residual Oil Combustion	050-995-1500-0000 	83071

Industrial Stationary IC Engines - Other Fuel	050-040-0012-0000	82107

Industrial LPG Combustion	050-995-0120-0000 	66795

Industrial Stationary IC Engines - Natural Gas	050-040-0110-0000	66787

Industrial Natural Gas Combustion (Unspecified) 	050-995-0110-0000 	47142

Commercial Distillate Oil Combustion	060-995-1220-0000 	47159

Commercial Residual Oil Combustion	060-995-1500-0000 	47183

Commercial LPG Combustion	060-995-0120-0000 	58727

Commercial Natural Gas Combustion - Space Heating	060-020-0110-0000 	58735

Commercial Natural Gas Combustion - Water Heating	060-030-0110-0000 	58743

Commercial Natural Gas Combustion (Unspecified)	060-995-0110-0000 	47167

Unspecified Combustion Sources 	099-995-0012-0000 	66837

## Liquid Fuels Combustion

### Emission Factors

Table 1 lists emission factors for commercial and industrial processes that commonly use liquid fuels, such as diesel or distillate fuel, liquefied petroleum gases (LPG), and residual fuel.  The combustion processes that are represented in Table 1 include IC engines, turbines, and boilers.

Table 1.   Emission factors for commercial and industrial processes that employ liquid fuels

(EPA, 2000a; EPA 1999a; EPA 1996a; EPA 1996b).

|  | Emission Factorsa (lb/thousand gallons fuel consumption) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fuel Type and Process | SOxb | NOx | CO | TOC | VOC | PMc | PM10d |
| Diesel fuel no. 2; IC engine (<600 hp)e | 39 | 604 | 130 | 49 | 42 | 42 | 42 |
| Distillate fuel no. 2; uncontrolled turbine | 42 | 121 | 0.45 | 0.55 | 0.056 | 1.6 | 1.6 |
| Distillate fuel no. 2; Water-steam injection controlled turbine | 42 | 33 | 9.9 | 0.55 | 0.056 | 1.6 | 1.6 |
| Distillate oil; Industrial Boiler | 49 | 10 | 5 | 0.252 | 0.2 | 3.3 | 1.65 |
| Distillate oil; Commercial Boiler | 49 | 10 | 5 | 0.556 | 0.34 | 3.3 | 1.82 |
| Residual fuel; Industrial boiler | 390 | 40 | 5 | 1.28 | 0.28 | 26.8 | 16.6 |
| Residual fuel; Commercial boiler | 390 | 40 | 5 | 1.605 | 1.13 | 26.8 | 16.6 |
| LPG; industrial boiler | 1.5 | 19 | 3.2 | 0.5 | 0.3 | 0.6 | 0.6 |
| LPG; commercial boiler | 4.5 | 14 | 1.9 | 0.5 | 0.3 | 0.4 | 0.4 |

For emission factor selection, it was assumed that most diesel and residual fuels are graded No. 2, and most residual fuels are graded No. 6.

a For diesel and distillate fuels, units were converted from lb/MMBtu to lb/thousand gal by using the following conversion rates: 0.0193 MMBtu/lb  7.1 lb/gal  1000 gal/thousand gal = 137 MMBtu/thousand gal

b Where applicable, SOx = SO2  + SO3.  The following sulfur contents were assumed: 0.3% for distillate oil; 2.4% for residual oil; 0.05% for diesel; 15 gr/100 ft3 for LPG.

c Where applicable, Total PM = filterable PM + condensable PM.

d The following mass ratios of PM10:Total PM were assumed:  0.5 for distillate/industrial; 0.55 for distillate/commercial; 0.62 for residual; 1.0 for LPG.

e The VOC emission factor for diesel IC engines is based on the EPA’s speciation profile for this source (weight percents 11.6% methane, 2.8% ethane) (EPA, 199b) and the assumption that so VOC equals non-methane non-ethane hydrocarbons.

### Fuel Consumption Data

Statewide fuel consumption data by fuel type is available for broad, comprehensive commercial and industrial sectors (EIA, 2001a).  Commercial fuel consumption was estimated for commercial buildings and public or private service-related entities. Industrial fuel consumption was estimated for manufacturing and other industries (i.e., mining, construction, agriculture, and forestry, and non-utility power generation). In 1999, the distributions of statewide commercial and industrial fuels consumption by fuel type were reported as follows.

- Commercial

- Distillate oil 1,654 thousand barrels (includes diesel)

- LPG 1,008 thousand barrels

- Residual fuel 0 barrels

- Industrial

- Distillate oil 8,737 thousand barrels (includes diesel)

- LPG 5,068 thousand barrels

- Residual fuel 684 thousand barrels

The EIA also collects fuel consumption data for manufacturing sectors in each major Census Region every four years (EIA, 2001b).  The most recent data available are from 1998.  The proportions of total industrial fuels that are consumed in the West Census Region by the manufacturing sector (excluding food processing and refinery production) are estimated below.

- Manufacturing (excluding food processing and refinery production) proportion of Total Industrial fuel consumption

- Distillate oil: 9 of 1,321 trillion Btu = 0.68%

- LPG: 3 of 90 trillion Btu manufacturing = 3.3%

- Residual fuel: 4 of 302 trillion Btu manufacturing = 1.3%

It is difficult to assess accurately the specific processes that are employed within these sectors.  In the commercial sector, energy is consumed for space and water heating, cooling, lighting, and miscellaneous processes.  In the industrial sector, energy is consumed largely for process heat and process operations.  In order to estimate the fuel consumption rates for specific fuel combustion processes, a few rough approximations must be applied.

- Diesel fuel (a type of distillate fuel) is combusted in internal combustion engines.

- Distillate fuel oil is combusted in boilers/heaters/burners or turbines.

- Residual fuel oils are burned in boilers/heaters/burners.

- Other than motor-vehicle LPG (HD-5), commercial-grade LPG is burned in boilers/heaters/burners.

An EIA survey (EIA, 1998) for the Pacific Census Region showed that in commercial buildings, 60% of fuel oil is used for water or space heating and 40% is used for miscellaneous applications. It is assumed that water and space heating processes are powered by boilers/heaters/burners, and that miscellaneous applications are powered by a combination of distillate-fired turbines and diesel-fired IC engines.  For miscellaneous applications we must make an arbitrary decision for the split in fuel consumption between turbines and IC engines.  (A 50-50% split was assumed.)  In addition, we will assume that commercial turbines are not equipped with NOx control devices.

In the manufacturing sector in 1994, 28% of distillate and 71% of residual were used as boiler fuel, 34% of distillate was used for heating, and 23% of distillate was used for on-site transportation (EIA, 1997a), which should be excluded from inclusion in this emissions source category.  It is assumed that the remaining 29% of residual fuel is used for heating and the remaining 15% of distillate is used to power stationary IC engines.

Emissions for industrial and commercial fuel consumption can be apportioned to the county level according to industrial and commercial employment (US Census Bureau, 2001).  For the sample calculations shown here, the manufacturing sector (NAICS 31) represents industrial employment, and the employment categories listed below represent commercial and services employment.

- Wholesale trade, NAICS 42

- Retail trade, NAICS 44

- Information, NAICS 51

- Finance and insurance, NAICS 52

- Real estate and rental and leasing, NAICS 53

- Professional, scientific and technical services, NAICS 54

- Management of companies and enterprises, NAICS 55

- Admin, support, waste management, remediation services, NAICS 56

- Educational services, NAICS 61

- Health care and social assistance, NAICS 62

- Arts, entertainment and recreation, NAICS 71

- Accommodation and food services, NAICS 72

- Other services (except public administration), NAICS 81

- Auxiliaries (except corporate, subsidiary and regional management), NAICS 95

- Unclassified establishments, NAICS 99

### Seasonal Allocation

Seasonal patterns of residual and distillate fuel were gathered from monthly fuel consumption reports (EIA, 2000/2001).  (Similar data for LPG were not available.)  It was determined that the consumption rates of these fuels remain fairly constant over the course of the year.  A flat seasonal distribution is recommended.

### Sample Calculation

The following is a sample calculation for combustion of distillate fuel in the industrial sector for Placer County (EIC code 050-995-1220-0000).  Statewide consumption of distillate fuel is disaggregated by process and the appropriate emission factors are applied.

8737 thousand barrels  0.68%   (28% + 34%) = 37 thousand barrels consumed in boilers or heaters

37 thousand barrels  42 gallons/barrel  10 lb NOx/thousand gal  2000 lb/ton = 8 tons

8737  0.68%  15% = 9 thousand barrels consumed in IC engines

9 thousand bbl  42 gal/bbl  604 lb NOx/thousand gal  2000 lb/ton = 114 tons

The sum of emissions from combustion in boilers, heaters, and IC engines is 122 tons.  Of total statewide manufacturing employment (1.8 million), 0.5 % (or 8 thousand) are employed in Placer County.

0.0045   122 tons = 0.5 tons

Tables 2, 3, and 4 tabulate the results of similar calculations for California and all of its counties.

Table 2.   Statewide liquid fuels consumption and emissions for the manufacturing and commercial sectors.

| Sector, Fuel Type, and Process Type | Proportion | Consumption (1000 bbl) | Consumption (1000 gal) | Emissions (tpy) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | SOx | NOx | CO | TOC | VOC | PM | PM10 |
| Manufacturing |  |  |  |  |  |  |  |  |  |  |
| Distillate |  |  |  |  |  |  |  |  |  |  |
| IC Engines | 0.15 | 9 | 375 | 7.3 | 113.3 | 24.4 | 9.2 | 7.9 | 7.9 | 7.9 |
| Turbines | 0 | 0 | 0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Boilers & Heaters | 0.62 | 37 | 1,550 | 38.0 | 7.8 | 3.9 | 0.2 | 0.2 | 2.6 | 1.3 |
| Residual |  |  |  |  |  |  |  |  |  |  |
| Boilers & Heaters | 1.0 | 9 | 381 | 74.2 | 7.6 | 1.0 | 0.2 | 0.1 | 5.1 | 3.2 |
| LPG |  |  |  |  |  |  |  |  |  |  |
| Boilers | 1.0 | 170 | 7,127 | 5.3 | 67.7 | 11.4 | 1.8 | 1.1 | 2.1 | 2.1 |
| Subtotal of Emissions for Manufacturing: |  |  |  | 124.9 | 196.3 | 40.6 | 11.4 | 9.2 | 17.7 | 14.5 |
| Commercial |  |  |  |  |  |  |  |  |  |  |
| Distillate |  |  |  |  |  |  |  |  |  |  |
| IC Engines | 0.2 | 331 | 13,894 | 270.9 | 4,195.9 | 903.1 | 340.4 | 291.8 | 291.8 | 291.8 |
| Turbines | 0.2 | 331 | 13,894 | 291.8 | 17.7 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| Boilers | 0.6 | 992 | 41,681 | 1,021.2 | 208.4 | 104.2 | 11.6 | 7.1 | 68.8 | 37.9 |
| Residual |  |  |  |  |  |  |  |  |  |  |
| Boilers | 1.0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| LPG |  |  |  |  |  |  |  |  |  |  |
| Boilers | 1.0 | 1,008 | 42,336 | 95.3 | 296.4 | 40.2 | 10.6 | 6.4 | 8.5 | 8.5 |
| Subtotal of Emissions for Commercial: |  |  |  | 1,679.1 | 4,718.3 | 1,047.5 | 362.6 | 305.2 | 369.0 | 338.2 |
| Grand Total of Emissions: |  |  |  | 1,804.0 | 4,914.6 | 1,088.1 | 374.0 | 314.4 | 386.7 | 352.6 |

| Table 3. Estimated county-specific emissions for industrial (manufacturing) liquid fuels combustion. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| County | Manufacturing Employment | Proportion of Statewide Employment | Total Emissions (tons per year) |  |  |  |  |  |  |  |
|  |  |  | SOx | NOx | CO | TOC | VOC | PM | PM10 |  |
| Alameda | 89,281 | 4.98% | 6.2 | 9.8 | 2.0 | 0.6 | 0.5 | 0.9 | 0.7 |  |
| Alpine | - | 0.00% | - | - | - | - | - | - | - |  |
| Amador | 857 | 0.05% | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Butte | 4,512 | 0.25% | 0.3 | 0.5 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Calaveras | 468 | 0.03% | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Colusa | 709 | 0.04% | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Contra Costa | 18,890 | 1.05% | 1.3 | 2.1 | 0.4 | 0.1 | 0.1 | 0.2 | 0.2 |  |
| Del Norte | 250 | 0.01% | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| El Dorado | 2,042 | 0.11% | 0.1 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Fresno | 26,406 | 1.47% | 1.8 | 2.9 | 0.6 | 0.2 | 0.1 | 0.3 | 0.2 |  |
| Glenn | 905 | 0.05% | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Humboldt | 5,262 | 0.29% | 0.4 | 0.6 | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 |  |
| Imperial | 1,506 | 0.08% | 0.1 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Inyo | 252 | 0.01% | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Kern | 13,140 | 0.73% | 0.9 | 1.4 | 0.3 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Kings | 2,948 | 0.16% | 0.2 | 0.3 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Lake | 338 | 0.02% | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Lassen | 325 | 0.02% | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Los Angeles | 622,885 | 34.76% | 43.4 | 68.2 | 14.1 | 4.0 | 3.2 | 6.1 | 5.0 |  |
| Madera | 3,964 | 0.22% | 0.3 | 0.4 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Marin | 4,227 | 0.24% | 0.3 | 0.5 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Mariposa | 113 | 0.01% | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Mendocino | 4,231 | 0.24% | 0.3 | 0.5 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Merced | 8,322 | 0.46% | 0.6 | 0.9 | 0.2 | 0.1 | 0.0 | 0.1 | 0.1 |  |
| Modoc | - | 0.00% | - | - | - | - | - | - | - |  |
| Mono | 39 | 0.00% | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Monterey | 6,755 | 0.38% | 0.5 | 0.7 | 0.2 | 0.0 | 0.0 | 0.1 | 0.1 |  |
| Napa | 10,019 | 0.56% | 0.7 | 1.1 | 0.2 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Nevada | 2,891 | 0.16% | 0.2 | 0.3 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Orange | 224,520 | 12.53% | 15.6 | 24.6 | 5.1 | 1.4 | 1.1 | 2.2 | 1.8 |  |
| Placer | 8,131 | 0.45% | 0.6 | 0.9 | 0.2 | 0.1 | 0.0 | 0.1 | 0.1 |  |
| Plumas | 642 | 0.04% | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Riverside | 49,509 | 2.76% | 3.5 | 5.4 | 1.1 | 0.3 | 0.3 | 0.5 | 0.4 |  |
| Sacramento | 31,865 | 1.78% | 2.2 | 3.5 | 0.7 | 0.2 | 0.2 | 0.3 | 0.3 |  |
| San Benito | 2,197 | 0.12% | 0.2 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| San Bernardino | 68,909 | 3.85% | 4.8 | 7.5 | 1.6 | 0.4 | 0.4 | 0.7 | 0.6 |  |
| San Diego | 116,648 | 6.51% | 8.1 | 12.8 | 2.6 | 0.7 | 0.6 | 1.2 | 0.9 |  |
| San Francisco | 21,725 | 1.21% | 1.5 | 2.4 | 0.5 | 0.1 | 0.1 | 0.2 | 0.2 |  |
| San Joaquin | 22,673 | 1.27% | 1.6 | 2.5 | 0.5 | 0.1 | 0.1 | 0.2 | 0.2 |  |
| San Luis Obispo | 6,894 | 0.38% | 0.5 | 0.8 | 0.2 | 0.0 | 0.0 | 0.1 | 0.1 |  |
| San Mateo | 31,803 | 1.77% | 2.2 | 3.5 | 0.7 | 0.2 | 0.2 | 0.3 | 0.3 |  |
| Santa Barbara | 16,464 | 0.92% | 1.1 | 1.8 | 0.4 | 0.1 | 0.1 | 0.2 | 0.1 |  |
| Santa Clara | 231,338 | 12.91% | 16.1 | 25.3 | 5.2 | 1.5 | 1.2 | 2.3 | 1.9 |  |
| Santa Cruz | 8,685 | 0.48% | 0.6 | 1.0 | 0.2 | 0.1 | 0.0 | 0.1 | 0.1 |  |
| Shasta | 3,555 | 0.20% | 0.2 | 0.4 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Sierra | 74 | 0.00% | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Siskiyou | 864 | 0.05% | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Solano | 9,202 | 0.51% | 0.6 | 1.0 | 0.2 | 0.1 | 0.0 | 0.1 | 0.1 |  |
| Sonoma | 26,391 | 1.47% | 1.8 | 2.9 | 0.6 | 0.2 | 0.1 | 0.3 | 0.2 |  |
| Stanislaus | 23,307 | 1.30% | 1.6 | 2.6 | 0.5 | 0.1 | 0.1 | 0.2 | 0.2 |  |
| Sutter | 1,660 | 0.09% | 0.1 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Tehama | 2,186 | 0.12% | 0.2 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Trinity | 197 | 0.01% | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Tulare | 12,048 | 0.67% | 0.8 | 1.3 | 0.3 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Tuolumne | 930 | 0.05% | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Ventura | 30,860 | 1.72% | 2.2 | 3.4 | 0.7 | 0.2 | 0.2 | 0.3 | 0.2 |  |
| Yolo | 6,034 | 0.34% | 0.4 | 0.7 | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 |  |
| Yuba | 1,213 | 0.07% | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| California | 1,792,061 | 100% | 125 | 196 | 41 | 11 | 9 | 18 | 14 |  |

| Table 4. Estimated county-specific emissions for commercial liquid fuels combustion. |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| County | Commercial Employment | Proportion of Statewide Employment | Total Emissions (tons per year) |  |  |  |  |  |  |  |
|  |  |  | SOx | NOx | CO | TOC | VOC | PM | PM10 |  |
| Alameda | 452,030 | 4.83% | 81.1 | 227.9 | 50.6 | 17.5 | 14.7 | 17.8 | 16.3 |  |
| Alpine | 460 | 0.00% | 0.1 | 0.2 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Amador | 5,678 | 0.06% | 1.0 | 2.9 | 0.6 | 0.2 | 0.2 | 0.2 | 0.2 |  |
| Butte | 43,182 | 0.46% | 7.7 | 21.8 | 4.8 | 1.7 | 1.4 | 1.7 | 1.6 |  |
| Calaveras | 4,151 | 0.04% | 0.7 | 2.1 | 0.5 | 0.2 | 0.1 | 0.2 | 0.2 |  |
| Colusa | 2,258 | 0.02% | 0.4 | 1.1 | 0.3 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Contra Costa | 250,885 | 2.68% | 45.0 | 126.5 | 28.1 | 9.7 | 8.2 | 9.9 | 9.1 |  |
| Del Norte | 3,722 | 0.04% | 0.7 | 1.9 | 0.4 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| El Dorado | 29,564 | 0.32% | 5.3 | 14.9 | 3.3 | 1.1 | 1.0 | 1.2 | 1.1 |  |
| Fresno | 154,720 | 1.65% | 27.8 | 78.0 | 17.3 | 6.0 | 5.0 | 6.1 | 5.6 |  |
| Glenn | 2,516 | 0.03% | 0.5 | 1.3 | 0.3 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Humboldt | 27,681 | 0.30% | 5.0 | 14.0 | 3.1 | 1.1 | 0.9 | 1.1 | 1.0 |  |
| Imperial | 18,456 | 0.20% | 3.3 | 9.3 | 2.1 | 0.7 | 0.6 | 0.7 | 0.7 |  |
| Inyo | 5,540 | 0.06% | 1.0 | 2.8 | 0.6 | 0.2 | 0.2 | 0.2 | 0.2 |  |
| Kern | 110,162 | 1.18% | 19.8 | 55.5 | 12.3 | 4.3 | 3.6 | 4.3 | 4.0 |  |
| Kings | 12,863 | 0.14% | 2.3 | 6.5 | 1.4 | 0.5 | 0.4 | 0.5 | 0.5 |  |
| Lake | 7,690 | 0.08% | 1.4 | 3.9 | 0.9 | 0.3 | 0.3 | 0.3 | 0.3 |  |
| Lassen | 3,261 | 0.03% | 0.6 | 1.6 | 0.4 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Los Angeles | 2,836,373 | 30.31% | 509.0 | 1,430.2 | 317.5 | 109.9 | 92.5 | 111.8 | 102.5 |  |
| Madera | 16,918 | 0.18% | 3.0 | 8.5 | 1.9 | 0.7 | 0.6 | 0.7 | 0.6 |  |
| Marin | 88,032 | 0.94% | 15.8 | 44.4 | 9.9 | 3.4 | 2.9 | 3.5 | 3.2 |  |
| Mariposa | 2,718 | 0.03% | 0.5 | 1.4 | 0.3 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Mendocino | 17,668 | 0.19% | 3.2 | 8.9 | 2.0 | 0.7 | 0.6 | 0.7 | 0.6 |  |
| Merced | 24,902 | 0.27% | 4.5 | 12.6 | 2.8 | 1.0 | 0.8 | 1.0 | 0.9 |  |
| Modoc | 1,331 | 0.01% | 0.2 | 0.7 | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 |  |
| Mono | 5,371 | 0.06% | 1.0 | 2.7 | 0.6 | 0.2 | 0.2 | 0.2 | 0.2 |  |
| Monterey | 84,754 | 0.91% | 15.2 | 42.7 | 9.5 | 3.3 | 2.8 | 3.3 | 3.1 |  |
| Napa | 35,610 | 0.38% | 6.4 | 18.0 | 4.0 | 1.4 | 1.2 | 1.4 | 1.3 |  |
| Nevada | 19,538 | 0.21% | 3.5 | 9.9 | 2.2 | 0.8 | 0.6 | 0.8 | 0.7 |  |
| Orange | 989,521 | 10.57% | 177.6 | 498.9 | 110.8 | 38.3 | 32.3 | 39.0 | 35.8 |  |
| Placer | 67,712 | 0.72% | 12.2 | 34.1 | 7.6 | 2.6 | 2.2 | 2.7 | 2.4 |  |
| Plumas | 3,218 | 0.03% | 0.6 | 1.6 | 0.4 | 0.1 | 0.1 | 0.1 | 0.1 |  |
| Riverside | 260,101 | 2.78% | 46.7 | 131.1 | 29.1 | 10.1 | 8.5 | 10.3 | 9.4 |  |
| Sacramento | 331,967 | 3.55% | 59.6 | 167.4 | 37.2 | 12.9 | 10.8 | 13.1 | 12.0 |  |
| San Benito | 6,251 | 0.07% | 1.1 | 3.2 | 0.7 | 0.2 | 0.2 | 0.2 | 0.2 |  |
| San Bernardino | 310,056 | 3.31% | 55.6 | 156.3 | 34.7 | 12.0 | 10.1 | 12.2 | 11.2 |  |
| San Diego | 806,152 | 8.62% | 144.7 | 406.5 | 90.2 | 31.2 | 26.3 | 31.8 | 29.1 |  |
| San Francisco | 465,946 | 4.98% | 83.6 | 234.9 | 52.2 | 18.1 | 15.2 | 18.4 | 16.8 |  |
| San Joaquin | 104,407 | 1.12% | 18.7 | 52.6 | 11.7 | 4.0 | 3.4 | 4.1 | 3.8 |  |
| San Luis Obispo | 56,300 | 0.60% | 10.1 | 28.4 | 6.3 | 2.2 | 1.8 | 2.2 | 2.0 |  |
| San Mateo | 261,403 | 2.79% | 46.9 | 131.8 | 29.3 | 10.1 | 8.5 | 10.3 | 9.4 |  |
| Santa Barbara | 106,785 | 1.14% | 19.2 | 53.8 | 12.0 | 4.1 | 3.5 | 4.2 | 3.9 |  |
| Santa Clara | 650,685 | 6.95% | 116.8 | 328.1 | 72.8 | 25.2 | 21.2 | 25.7 | 23.5 |  |
| Santa Cruz | 59,932 | 0.64% | 10.8 | 30.2 | 6.7 | 2.3 | 2.0 | 2.4 | 2.2 |  |
| Shasta | 34,943 | 0.37% | 6.3 | 17.6 | 3.9 | 1.4 | 1.1 | 1.4 | 1.3 |  |
| Sierra | 279 | 0.00% | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Siskiyou | 7,120 | 0.08% | 1.3 | 3.6 | 0.8 | 0.3 | 0.2 | 0.3 | 0.3 |  |
| Solano | 67,543 | 0.72% | 12.1 | 34.1 | 7.6 | 2.6 | 2.2 | 2.7 | 2.4 |  |
| Sonoma | 116,926 | 1.25% | 21.0 | 59.0 | 13.1 | 4.5 | 3.8 | 4.6 | 4.2 |  |
| Stanislaus | 80,524 | 0.86% | 14.4 | 40.6 | 9.0 | 3.1 | 2.6 | 3.2 | 2.9 |  |
| Sutter | 12,768 | 0.14% | 2.3 | 6.4 | 1.4 | 0.5 | 0.4 | 0.5 | 0.5 |  |
| Tehama | 7,500 | 0.08% | 1.3 | 3.8 | 0.8 | 0.3 | 0.2 | 0.3 | 0.3 |  |
| Trinity | 1,143 | 0.01% | 0.2 | 0.6 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |  |
| Tulare | 50,797 | 0.54% | 9.1 | 25.6 | 5.7 | 2.0 | 1.7 | 2.0 | 1.8 |  |
| Tuolumne | 9,782 | 0.10% | 1.8 | 4.9 | 1.1 | 0.4 | 0.3 | 0.4 | 0.4 |  |
| Ventura | 171,557 | 1.83% | 30.8 | 86.5 | 19.2 | 6.6 | 5.6 | 6.8 | 6.2 |  |
| Yolo | 41,530 | 0.44% | 7.5 | 20.9 | 4.6 | 1.6 | 1.4 | 1.6 | 1.5 |  |
| Yuba | 6,650 | 0.07% | 1.2 | 3.4 | 0.7 | 0.3 | 0.2 | 0.3 | 0.2 |  |
| California | 9,357,532 | 100% | 1,679 | 4,718 | 1,048 | 363 | 305 | 369 | 338 |  |

## Natural Gas Combustion

### Emissions from Industrial Natural Gas Combustion

Industrial facilities combust natural gas in boilers, turbines, or engines to generate power and heat for production and miscellaneous processes.  Table 5 lists emission factors for various natural gas combustion processes.

Table 5.  Emission factors used to calculate the emissions generated by large industrial boilers and turbines for natural gas fuel (EPA, 2000a; EPA, 2000b; EPA, 1998).

|  | Emission Factors a (lb/MMscf) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Process | SO2 b | NOx | CO | TOC | VOC | Total PM |
| Large Boiler: Uncontrolled | 0.6 | 190 | 84 | 11 | 5.5 | 7.6 |
| Large Boiler: Low NOx Burner | 0.6 | 140 | 84 | 11 | 5.5 | 7.6 |
| Large Boiler: Flue Gas Recirculation | 0.6 | 100 | 84 | 11 | 5.5 | 7.6 |
| Turbine: Uncontrolledc | 0.3 | 326 | 83.6 | 11.2 | 2.1 | 6.7 |
| Turbine: Water-Steam Injectedc | 0.3 | 132 | 30.6 | 11.2 | 2.1 | 6.7 |

a Units are expressed in of pounds of pollutant per million standard cubic feet natural gas.

b Assumes gas H2S content of 3.18 ppmv.

c  Assumes natural gas heating value of 1020 Btu/scf.

#### Activity Data

The largest consumers of industrial natural gas are chemical (38% of consumption), petroleum (12%), primary metal (12%), food processing (9%), paper (8%), stone/clay/glass (6%), and fabricated metal (3%) manufacturers (EIA, 1997b).  In the industrial sector, 42% of natural gas is consumed for process uses (most of it for process heating), 35% for boiler fuel, 12% for non-process use (50-50% split between space heating and electricity generation), and 10% used as feedstock (EIA, 1997b).  Therefore, its is assumed that 84% of industrial natural gas is consumed in boilers/heaters/burners, 6% is consumed in natural gas turbines, and 10% (feedstock) is not associated with combustion emissions.

For many areas of California, the amount of natural gas consumed by the industrial sector is available from Pacific Gas and Electric (PG&E) and can be obtained by contacting their Resource Center (telephone number 415-973-7206.  However, we have opted to use estimates of natural gas consumption that were prepared by the Demand Analysis Office of the California Energy Commission (CEC) (telephone 916-654-4755) because they were able to meet our request for information relatively quickly.  In addition, the CEC segregates consumption data into a more detailed set of categories such that Electricity Generation is a separate category from Industry.  Thus, it seems that for counties with electric power generation, the CEC consumption estimates are more accurate.  For year 2000, the California Energy Commission forecast the statewide industrial consumption of natural gas to be 6,190 million therms (or 606,900 million standard cubic feet).

#### Temporal Distribution

Industrial natural gas consumption varies throughout the year.  EIA reports monthly natural gas consumption by California’s industrial facilities (EIA, 2001c).  The reported monthly distribution is recommended for use to apportion annual emissions.  The recommended monthly apportionment scheme is summarized below.  No information is available to estimate diurnal profiles for this category.

January	7.1%

February	6.4%

March	6.4%

April	6.1%

May	7.9%

June	9.0%

July	9.9%

August	11.5%

September	9.6%

October	10.0%

November	8.2%

December	8.0%

#### Example Calculation

PG&E forecast El Dorado County’s industrial natural gas consumption to be 160 MMscf in year 2000.  It is estimated that 84% (or 134 MMscf) of this is combusted in boilers/heaters/burners and that 6% (or 10 MMscf) is combusted in natural gas turbines.  In addition, it is assumed that industrial consumers of natural gas employ NOx emissions control technologies.  Emission factors are selected and applied as shown below.

134 MMscf  84 lb CO/MMscf 2000 lb/ton = 5.6 tons per year CO (boilers)

10 MMscf  30.6 lb CO/MMscf  2000 lb/ton = 0.15 tons per year CO (turbines)

Table 6 tabulates the results of similar calculations for all of California’s counties.

| Table 6. Estimated industrial natural gas consumption and annual emissions. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Total Emissions (tons per year) |  |  |  |  |  |
| County | Industrial Natural Gas Consumption (MMscf) | Consumption in Boilers (MMscf) | Consumption in Turbines (MMscf) | NOx | SOx | CO | TOC | VOC | PM |
| Alameda | 12,465 | 10,471 | 748 | 678 | 3 | 451 | 62 | 30 | 42 |
| Alpine | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Amador | 431 | 362 | 26 | 23 | 0 | 16 | 2 | 1 | 1 |
| Butte | 946 | 795 | 57 | 51 | 0 | 34 | 5 | 2 | 3 |
| Calaveras | 2 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Colusa | 432 | 363 | 26 | 23 | 0 | 16 | 2 | 1 | 1 |
| Contra Costa | 97,301 | 81,733 | 5,838 | 5,289 | 25 | 3,522 | 482 | 231 | 330 |
| Del Norte | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| El Dorado | 160 | 134 | 10 | 9 | 0 | 6 | 1 | 0 | 1 |
| Fresno | 14,898 | 12,514 | 894 | 810 | 4 | 539 | 74 | 35 | 51 |
| Glenn | 791 | 664 | 47 | 43 | 0 | 29 | 4 | 2 | 3 |
| Humboldt | 1,851 | 1,555 | 111 | 101 | 0 | 67 | 9 | 4 | 6 |
| Imperial | 2,965 | 2,491 | 178 | 161 | 1 | 107 | 15 | 7 | 10 |
| Inyo | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Kern | 69,213 | 58,139 | 4,153 | 3,762 | 18 | 2,505 | 343 | 164 | 235 |
| Kings | 2,154 | 1,809 | 129 | 117 | 1 | 78 | 11 | 5 | 7 |
| Lake | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Lassen | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Los Angeles | 229,561 | 192,832 | 13,774 | 12,479 | 60 | 8,310 | 1,138 | 545 | 779 |
| Madera | 3,432 | 2,883 | 206 | 187 | 1 | 124 | 17 | 8 | 12 |
| Marin | 282 | 237 | 17 | 15 | 0 | 10 | 1 | 1 | 1 |
| Mariposa | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Mendocino | 1,575 | 1,323 | 95 | 86 | 0 | 57 | 8 | 4 | 5 |
| Merced | 5,279 | 4,435 | 317 | 287 | 1 | 191 | 26 | 13 | 18 |
| Modoc | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Mono | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Monterey | 1,788 | 1,502 | 107 | 97 | 0 | 65 | 9 | 4 | 6 |
| Napa | 323 | 272 | 19 | 18 | 0 | 12 | 2 | 1 | 1 |
| Nevada | 16 | 14 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |
| Orange | 15,880 | 13,339 | 953 | 863 | 4 | 575 | 79 | 38 | 54 |
| Placer | 1,242 | 1,044 | 75 | 68 | 0 | 45 | 6 | 3 | 4 |
| Plumas | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Riverside | 8,852 | 7,436 | 531 | 481 | 2 | 320 | 44 | 21 | 30 |
| Sacramento | 6,338 | 5,324 | 380 | 345 | 2 | 229 | 31 | 15 | 22 |
| San Benito | 910 | 765 | 55 | 49 | 0 | 33 | 5 | 2 | 3 |
| San Bernardino | 37,633 | 31,611 | 2,258 | 2,046 | 10 | 1,362 | 187 | 89 | 128 |
| San Diego | 4,913 | 4,127 | 295 | 267 | 1 | 178 | 24 | 12 | 17 |
| San Francisco | 582 | 489 | 35 | 32 | 0 | 21 | 3 | 1 | 2 |
| San Joaquin | 13,905 | 11,680 | 834 | 756 | 4 | 503 | 69 | 33 | 47 |
| San Luis Obispo | 1,465 | 1,231 | 88 | 80 | 0 | 53 | 7 | 3 | 5 |
| San Mateo | 1,551 | 1,302 | 93 | 84 | 0 | 56 | 8 | 4 | 5 |
| Santa Barbara | 5,111 | 4,294 | 307 | 278 | 1 | 185 | 25 | 12 | 17 |
| Santa Clara | 9,843 | 8,268 | 591 | 535 | 3 | 356 | 49 | 23 | 33 |
| Santa Cruz | 645 | 542 | 39 | 35 | 0 | 23 | 3 | 2 | 2 |
| Shasta | 4,511 | 3,789 | 271 | 245 | 1 | 163 | 22 | 11 | 15 |
| Sierra | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Siskiyou | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Solano | 8,649 | 7,265 | 519 | 470 | 2 | 313 | 43 | 21 | 29 |
| Sonoma | 904 | 759 | 54 | 49 | 0 | 33 | 4 | 2 | 3 |
| Stanislaus | 9,498 | 7,978 | 570 | 516 | 2 | 344 | 47 | 23 | 32 |
| Sutter | 526 | 442 | 32 | 29 | 0 | 19 | 3 | 1 | 2 |
| Tehama | 1,483 | 1,246 | 89 | 81 | 0 | 54 | 7 | 4 | 5 |
| Trinity | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Tulare | 5,493 | 4,614 | 330 | 299 | 1 | 199 | 27 | 13 | 19 |
| Tuolumne | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Ventura | 17,677 | 14,849 | 1,061 | 961 | 5 | 640 | 88 | 42 | 60 |
| Yolo | 3,196 | 2,684 | 192 | 174 | 1 | 116 | 16 | 8 | 11 |
| Yuba | 217 | 182 | 13 | 12 | 0 | 8 | 1 | 1 | 1 |
| Total | 606,890 | 509,788 | 36,413 | 32,991 | 158 | 21,968 | 3,008 | 1,440 | 2,059 |

### Emissions from Commercial Natural Gas Combustion

Commercial energy needs are met with a variety of natural gas-fired boilers, turbines, or internal combustion engines.  In general, natural gas for space heating and water heating is burned in natural gas-fired boilers.  After these uses, commercial cooking (which is part of the unspecified combustion category listed above) is the third largest end use of natural gas in California.

Cogeneration facilities recover thermal energy from the exhaust of electricity-generating turbines or internal combustion engines and use the recovered energy to generate steam for use in other processes, such as space heating, water heating, or industrial processes.  Many entities with cogeneration plants have significant electricity and heat energy demands (such as refineries, large factories, or independent power producers) and are most likely to be included in point source inventories.  However, small-capacity cogeneration is sometimes undertaken on by commercial entities.  Examples of facilities with potential to use micro-cogeneration devices are restaurants, hotels, residential health care facilities, small factories, health clubs, colleges, municipalities, hospitals, large apartment buildings, swimming pools, laundries, food and beverage processors, schools, and greenhouses.  Most cogeneration devices are natural gas powered, although it is feasible to design a cogeneration system that uses any other type of fuel (such as distillate oil, wood waste, etc.).  Smaller-scale cogenerators more often operate internal combustion engines, and larger-scale facilities more often run turbines.

In order to select and apply AP-42 emission factors, natural gas consumption (by use and combustion type) and the design parameters listed below must be determined.

Type	Parameters needed for emission factor selection

Boilers	Design heat input and control technology

Turbines	Control technology

Engines	Operating load, engine stroke (2- or 4-cycle), and fuel ratio (lean or rich burn).

The following steps and recommendations are provided as guidance for estimating emissions for commercial natural gas combustion.

Step 1:  Determine commercial natural gas fuel sales for the county.  A forecast for the year 2000, which was prepared in August 1998 (Table 7), was obtained from the Demand Analysis Office of the Information and Analysis Division at the California Energy Commission (telephone 916-654-4755).  Subtract from this figure the total natural gas consumption reported for any commercial facilities included in the point source inventory.  For Placer County, split commercial gas use between the Valley and Mountain Regions according to a spatially resolved surrogate, such as service/commercial employment by census tract.  Employment data for Placer Co. (Table 8) and a map of traffic analysis zones were obtained from Sacramento Area Council of Governments and the Tahoe Regional Planning Agency.  For other counties, these data are available from the local transportation planning agency or from Caltrans.  Year-2000 distributions were developed for ARB for the purposes of distributing area source surrogates.  These estimates are presented in Table 9.

Step 2:  Split commercial natural gas consumption according to end use (space heating, water heating, or unspecified).  Pacific Gas and Electric Company’s (PG&E) 1999 Commercial Building Survey Report was obtained from its Information Resource Center (telephone 415-973-7206).  This report summarizes the results of a comprehensive survey of all of PG&E’s commercial customers in the State of California.  Table 10 provides the typical distribution of natural gas end uses for commercial entities in four climate zones of California (see Figure 1).  Several counties are split between two climate zones.  Countywide natural gas usage should be distributed to each climate zone according to a spatially resolved surrogate, such as total employment by census tract (similar to the split suggested for Placer Co. in step 1 above).

Step 3:  Split consumption by end use according to combustion type (natural gas turbines, natural gas engines, or boilers).  No centralized source of information is available to consult for this information, therefore the following assumptions are recommended in the absence of county-specific information (e.g., telephone survey or other).

- Assume that commercial natural gas users accomplish space heating, water heating, and 60% of process uses with boilers of design heat input capacities between 0.3 and 10 MMBtu/hr and no emissions controls.  Assume that emissions due to commercial natural gas cooking operations can be approximated by applying the emission factors for boilers.

- Assume that commercial cooling, 40% of process uses, and other natural gas applications are achieved with natural gas-fired turbines or with internal combustion engines with a 4-cycle, lean burn design.

Step 4:  Select appropriate emission factors from AP-42 and apply them to estimates of fuel consumption.  The following assumptions are recommended for selection of emission factors.  For a conservatively high estimate, the higher emission factor for each pollutant should be chosen from those available.

- PM is essentially equal to PM10.

- PM10 includes filterable and condensable fractions.

- TOG equal total organic compounds (TOC).

- ROG equal TOC minus methane.

|  | Historical Consumption |  |  |  |  |  |  |  | Forecast Consumption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| COUNTY | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2004 | 2005 | 2006 | 2007 |
| ALAMEDA | 94 | 100 | 89 | 95 | 103 | 94 | 95 | 103 | 95 | 96 | 96 | 97 | 98 | 98 | 98 | 99 | 99 | 99 |
| ALPINE | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| AMADOR | 3 | 3 | 3 | 3 | 3 | 3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| BUTTE | 11 | 11 | 9 | 10 | 10 | 10 | 9 | 10 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 10 | 10 | 10 |
| CALAVERAS | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| COLUSA | 3 | 2 | 6 | 3 | 2 | 8 | 8 | 9 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 9 | 9 | 9 |
| CONTRA COSTA | 111 | 48 | 37 | 61 | 24 | 34 | 35 | 38 | 36 | 37 | 37 | 37 | 38 | 38 | 38 | 39 | 39 | 39 |
| DEL NORTE | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| EL DORADO | 5 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
| FRESNO | 44 | 44 | 39 | 44 | 59 | 55 | 44 | 48 | 43 | 44 | 45 | 45 | 45 | 46 | 46 | 47 | 47 | 47 |
| GLENN | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| HUMBOLDT | 10 | 10 | 8 | 9 | 9 | 9 | 25 | 27 | 23 | 24 | 24 | 24 | 25 | 25 | 25 | 25 | 25 | 25 |
| IMPERIAL | 1 | 1 | 1 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 |
| INYO | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| KERN | 36 | 42 | 50 | 16 | 13 | 15 | 14 | 14 | 14 | 14 | 14 | 14 | 14 | 15 | 15 | 15 | 15 | 15 |
| KINGS | 10 | 8 | 8 | 6 | 5 | 4 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| LAKE | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| LASSEN | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 5 | 19 | 18 | 18 | 18 | 19 | 18 | 18 | 18 |
| LOS ANGELES | 467 | 445 | 417 | 460 | 459 | 438 | 465 | 465 | 500 | 511 | 520 | 531 | 540 | 550 | 560 | 569 | 580 | 588 |
| MADERA | 6 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
| MARIN | 17 | 17 | 15 | 16 | 17 | 16 | 18 | 19 | 17 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 | 18 |
| MARIPOSA | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| MENDOCINO | 3 | 3 | 3 | 3 | 3 | 3 | 9 | 9 | 8 | 8 | 8 | 9 | 9 | 9 | 9 | 9 | 9 | 9 |
| MERCED | 14 | 9 | 8 | 6 | 16 | 14 | 14 | 15 | 13 | 14 | 14 | 14 | 14 | 14 | 14 | 14 | 14 | 15 |
| MODOC | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| MONO | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| MONTEREY | 30 | 29 | 26 | 29 | 28 | 27 | 38 | 41 | 40 | 40 | 41 | 41 | 42 | 42 | 43 | 43 | 43 | 44 |
| NAPA | 13 | 12 | 9 | 10 | 10 | 10 | 9 | 10 | 10 | 10 | 10 | 10 | 11 | 11 | 11 | 11 | 11 | 11 |
| NEVADA | 2 | 3 | 2 | 3 | 3 | 2 | 8 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 | 9 |
| ORANGE | 121 | 122 | 115 | 120 | 116 | 116 | 138 | 139 | 149 | 152 | 154 | 157 | 160 | 162 | 165 | 167 | 169 | 172 |
| PLACER | 9 | 9 | 9 | 10 | 10 | 10 | 15 | 16 | 16 | 17 | 17 | 17 | 17 | 18 | 18 | 18 | 18 | 18 |
| PLUMAS | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| RIVERSIDE | 48 | 47 | 45 | 50 | 52 | 50 | 56 | 56 | 62 | 64 | 66 | 68 | 70 | 72 | 74 | 76 | 77 | 79 |
| SACRAMENTO | 60 | 63 | 57 | 62 | 72 | 63 | 74 | 81 | 79 | 80 | 81 | 83 | 84 | 85 | 86 | 87 | 88 | 89 |
| SAN BENITO | 1 | 2 | 1 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 |
| SAN BERNARDINO | 117 | 81 | 76 | 85 | 101 | 74 | 75 | 75 | 82 | 85 | 88 | 91 | 93 | 96 | 98 | 101 | 103 | 106 |
| SAN DIEGO | 147 | 71 | 109 | 198 | 149 | 151 | 197 | 210 | 211 | 216 | 220 | 223 | 227 | 230 | 233 | 237 | 240 | 243 |
| SAN FRANCISCO | 96 | 101 | 86 | 93 | 102 | 97 | 99 | 108 | 98 | 99 | 99 | 100 | 100 | 101 | 101 | 101 | 101 | 101 |
| SAN JOAQUIN | 34 | 28 | 28 | 29 | 32 | 34 | 23 | 25 | 24 | 24 | 24 | 25 | 25 | 25 | 25 | 26 | 26 | 26 |
| SAN LUIS OBISPO | 16 | 16 | 16 | 16 | 17 | 15 | 16 | 17 | 16 | 17 | 17 | 17 | 17 | 17 | 17 | 18 | 18 | 18 |
| SAN MATEO | 47 | 50 | 45 | 47 | 53 | 50 | 53 | 58 | 53 | 53 | 53 | 54 | 54 | 54 | 54 | 54 | 55 | 54 |
| SANTA BARBARA | 34 | 24 | 23 | 23 | 28 | 30 | 23 | 24 | 25 | 25 | 26 | 26 | 27 | 27 | 27 | 28 | 28 | 28 |
| SANTA CLARA | 117 | 120 | 101 | 110 | 121 | 115 | 103 | 112 | 108 | 110 | 111 | 113 | 114 | 115 | 116 | 117 | 118 | 119 |
| SANTA CRUZ | 16 | 26 | 22 | 16 | 16 | 25 | 26 | 29 | 26 | 26 | 26 | 27 | 27 | 27 | 27 | 27 | 27 | 27 |
| SHASTA | 10 | 7 | 15 | 9 | 32 | 23 | 11 | 13 | 11 | 11 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| SIERRA | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| SISKIYOU | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| SOLANO | 18 | 18 | 15 | 16 | 17 | 16 | 15 | 16 | 16 | 16 | 16 | 16 | 17 | 17 | 17 | 17 | 17 | 17 |
| SONOMA | 22 | 24 | 23 | 24 | 25 | 24 | 22 | 24 | 23 | 23 | 23 | 24 | 24 | 24 | 24 | 25 | 25 | 25 |
| STANISLAUS | 18 | 19 | 19 | 20 | 20 | 25 | 21 | 23 | 21 | 21 | 22 | 22 | 22 | 22 | 22 | 23 | 23 | 23 |
| SUTTER | 5 | 12 | 6 | 5 | 8 | 6 | 5 | 6 | 5 | 5 | 5 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| TEHAMA | 3 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| TRINITY | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| TULARE | 17 | 15 | 15 | 16 | 16 | 16 | 14 | 14 | 15 | 15 | 16 | 16 | 16 | 16 | 17 | 17 | 17 | 17 |
| TUOLUMNE | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| VENTURA | 36 | 49 | 40 | 23 | 23 | 29 | 28 | 28 | 30 | 31 | 32 | 32 | 33 | 33 | 34 | 34 | 35 | 35 |
| YOLO | 19 | 18 | 23 | 20 | 22 | 21 | 24 | 26 | 25 | 25 | 25 | 26 | 26 | 26 | 26 | 27 | 27 | 27 |
| YUBA | 4 | 4 | 4 | 4 | 4 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| Total | 1,895 | 1,720 | 1,631 | 1,784 | 1,815 | 1,749 | 1,852 | 1,945 | 1,946 | 1,986 | 2,031 | 2,062 | 2,093 | 2,120 | 2,148 | 2,174 | 2,202 | 2,223 |

| SACOG/TRPA Traffic Analysis Zone | Census Tract | Regional Analysis District | Total Employment | PG&E Climate Zone |
| --- | --- | --- | --- | --- |
| 157 | 202 | Foresthill | 6 | Mountain |
| 966 | 202 | Foresthill | 53 | Mountain |
| 977 | 202 | Foresthill | 175 | Mountain |
| 1057 | 220 | Colfax | 745 | Mountain |
| 1058 | 220 | Colfax | 493 | Mountain |
| 1059 | 202 | Placer High Country | 266 | Mountain |
| 1060 | 202 | Placer High Country | 0 | Mountain |
| 1061 | 220 | Placer High Country | 1162 | Mountain |
| 158 | 203 | Auburn | 263 | Valley |
| 159 | 204 | Auburn | 789 | Valley |
| 160 | 205 | Auburn | 347 | Valley |
| 161 | 206 | Granite Bay | 449 | Valley |
| 162 | 206 | Loomis | 236 | Valley |
| 163 | 206 | Granite Bay | 775 | Valley |
| 164 | 207 | Roseville | 844 | Valley |
| 165 | 207 | Roseville | 2931 | Valley |
| 166 | 207 | Roseville | 3472 | Valley |
| 167 | 207 | Roseville | 1263 | Valley |
| 168 | 207 | Roseville | 1916 | Valley |
| 169 | 207 | Roseville | 148 | Valley |
| 170 | 207 | Roseville | 244 | Valley |
| 171 | 207 | Roseville | 1080 | Valley |
| 172 | 207 | Roseville | 856 | Valley |
| 173 | 208 | Roseville | 655 | Valley |
| 174 | 208 | Roseville | 703 | Valley |
| 175 | 208 | Roseville | 1106 | Valley |
| 176 | 209 | Roseville | 2252 | Valley |
| 177 | 209 | Roseville | 1330 | Valley |
| 178 | 209 | Roseville | 281 | Valley |
| 179 | 209 | Roseville | 774 | Valley |
| 180 | 210 | Roseville | 1073 | Valley |
| 181 | 210 | Roseville | 618 | Valley |
| 182 | 210 | Roseville | 111 | Valley |
| 183 | 210 | Roseville | 368 | Valley |
| 184 | 210 | Roseville | 612 | Valley |
| 185 | 210 | Roseville | 578 | Valley |
| 186 | 210 | Roseville | 241 | Valley |
| 187 | 210 | Roseville | 2582 | Valley |
| 188 | 210 | Roseville | 4435 | Valley |
| 189 | 211 | Rocklin | 456 | Valley |
| 190 | 211 | Rocklin | 165 | Valley |
| 191 | 211 | Rocklin | 131 | Valley |
| 192 | 211 | Rocklin | 979 | Valley |
| 193 | 211 | Rocklin | 989 | Valley |
| 194 | 211 | Rocklin | 13 | Valley |
| 195 | 211 | Rocklin | 450 | Valley |
| 196 | 211 | Rocklin | 750 | Valley |
| 197 | 211 | Rocklin | 63 | Valley |
| 198 | 211 | Rocklin | 349 | Valley |
| 199 | 211 | Rocklin | 46 | Valley |
| 200 | 211 | Rocklin | 19 | Valley |
| 201 | 211 | Rocklin | 375 | Valley |
| 202 | 211 | Rocklin | 243 | Valley |
| 203 | 211 | Rocklin | 0 | Valley |
| 204 | 212 | Loomis | 795 | Valley |
| 205 | 212 | Loomis | 475 | Valley |
| 206 | 213 | Lincoln | 0 | Valley |
| 207 | 214 | Lincoln | 888 | Valley |
| 208 | 214 | Lincoln | 513 | Valley |
| 209 | 214 | Lincoln | 511 | Valley |
| 210 | 214 | Lincoln | 265 | Valley |
| 211 | 214 | Lincoln | 69 | Valley |
| 212 | 215 | Auburn | 452 | Valley |
| 213 | 216 | North Auburn | 1597 | Valley |
| 214 | 215 | Auburn | 3516 | Valley |
| 215 | 218 | North Auburn | 95 | Valley |
| 835 | 206 | Granite Bay | 191 | Valley |
| 836 | 206 | Loomis | 83 | Valley |
| 837 | 206 | Granite Bay | 526 | Valley |
| 838 | 213 | Rocklin | 186 | Valley |
| 839 | 213 | Rocklin | 0 | Valley |
| 840 | 213 | Sheridan | 60 | Valley |
| 841 | 213 | Lincoln | 36 | Valley |
| 842 | 213 | West Placer | 6 | Valley |
| 843 | 213 | Rocklin | 60 | Valley |
| 844 | 213 | Rocklin | 1433 | Valley |
| 845 | 213 | Lincoln | 1784 | Valley |
| 846 | 213 | Lincoln | 112 | Valley |
| 929 | 203 | Auburn | 638 | Valley |
| 930 | 203 | Auburn | 932 | Valley |
| 931 | 203 | Auburn | 914 | Valley |
| 932 | 203 | Auburn | 220 | Valley |
| 933 | 203 | Auburn | 327 | Valley |
| 934 | 203 | Auburn | 1094 | Valley |
| 935 | 203 | Auburn | 469 | Valley |
| 936 | 204 | Auburn | 173 | Valley |
| 937 | 204 | Auburn | 325 | Valley |
| 938 | 204 | Auburn | 961 | Valley |
| 939 | 204 | Auburn | 392 | Valley |
| 940 | 204 | Auburn | 811 | Valley |
| 941 | 204 | Auburn | 637 | Valley |
| 942 | 204 | Auburn | 912 | Valley |
| 943 | 205 | Auburn | 317 | Valley |
| 944 | 205 | Auburn | 24 | Valley |
| 945 | 205 | Auburn | 383 | Valley |
| 946 | 206 | Granite Bay | 125 | Valley |
| 947 | 207 | Roseville | 449 | Valley |
| 948 | 207 | Roseville | 698 | Valley |
| 949 | 207 | Roseville | 479 | Valley |
| 950 | 207 | Roseville | 723 | Valley |
| 951 | 208 | Roseville | 1562 | Valley |
| 952 | 208 | Roseville | 1529 | Valley |
| 953 | 209 | Roseville | 2048 | Valley |
| 954 | 209 | Roseville | 908 | Valley |
| 955 | 211 | Rocklin | 898 | Valley |
| 956 | 211 | Rocklin | 1437 | Valley |
| 957 | 211 | Rocklin | 2469 | Valley |
| 958 | 212 | Loomis | 951 | Valley |
| 960 | 215 | Auburn | 315 | Valley |
| 961 | 215 | Auburn | 804 | Valley |
| 962 | 215 | Auburn | 342 | Valley |
| 963 | 215 | Auburn | 47 | Valley |
| 964 | 215 | Auburn | 527 | Valley |
| 965 | 215 | Auburn | 256 | Valley |
| 967 | 216 | North Auburn | 35 | Valley |
| 968 | 216 | North Auburn | 45 | Valley |
| 969 | 218 | North Auburn | 40 | Valley |
| 970 | 218 | North Auburn | 95 | Valley |
| 971 | 218 | North Auburn | 41 | Valley |
| 972 | 218 | North Auburn | 198 | Valley |
| 973 | 218 | North Auburn | 3742 | Valley |
| 974 | 218 | North Auburn | 271 | Valley |
| 975 | 218 | North Auburn | 68 | Valley |
| 976 | 206 | Granite Bay | 573 | Valley |
| 978 | 213 | Rocklin | 295 | Valley |
| 979 | 213 | Sheridan | 90 | Valley |
| 980 | 213 | Sheridan | 21 | Valley |
| 981 | 213 | Lincoln | 70 | Valley |
| 982 | 213 | West Placer | 26 | Valley |
| 983 | 213 | West Placer | 12 | Valley |
| 984 | 213 | Rocklin | 1110 | Valley |
| 985 | 213 | Rocklin | 131 | Valley |
| 988 | 206 | Granite Bay | 374 | Valley |
| 1039 | 207 | Roseville | 1611 | Valley |
| 1040 | 207 | Roseville | 3296 | Valley |
| 1055 | 219 | Colfax | 214 | Valley |
| 1056 | 219 | Colfax | 319 | Valley |
| 1081 | 209 | Roseville | 464 | Valley |
| 1082 | 210 | Roseville | 423 | Valley |
| 1083 | 210 | Roseville | 2760 | Valley |
| 1084 | 210 | Roseville | 1100 | Valley |
| 1085 | 210 | Roseville | 85 | Valley |
| 1086 | 210 | Roseville | 175 | Valley |
| 1087 | 210 | Roseville | 6645 | Valley |
| 1088 | 210 | Roseville | 1003 | Valley |
| 1089 | 210 | Roseville | 1306 | Valley |
| 1090 | 210 | Roseville | 350 | Valley |
| 1091 | 213 | Lincoln | 3 | Valley |
| 1092 | 213 | Rocklin | 1220 | Valley |
| 1137 | 207 | Roseville | 2690 | Valley |
| TRPA Sum |  | Lake Tahoe Region | 4043 | Mountain |

Table 9.   Distributions of socioeconomic activities for counties that are split between air basins.

| County | Air Basin | Proportion of County Total Activity in Air Basin |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Total Population | Total Employment | Non-retail Employment | Industrial Employment | Service/ Commercial Employment |
| Solano | Sacramento Valley | 30.4% | 26.3% | 26.7% | 26.0% | 25.8% |
|  | San Francisco Bay Area | 69.6% | 73.7% | 73.3% | 74.0% | 74.2% |
| El Dorado | Lake Tahoe | 11.9% | 11.2% | 9.2% | 33.9% | 14.2% |
|  | Mountain Counties | 88.1% | 88.8% | 90.8% | 66.1% | 85.8% |
| Placer | Lake Tahoe | 2.7% | 1.9% | 1.8% | 6.1% | 1.5% |
|  | Mountain Counties | 10.9% | 3.6% | 3.4% | 0.3% | 3.4% |
|  | Sacramento Valley | 86.4% | 94.6% | 94.8% | 93.5% | 95.1% |

Table 10.   Distribution of annual natural gas sales to commercial PG&E customers for various

end uses in four PG&E climate zones.  (Source: PG&E 1999 Commercial Building Survey Report)

| PG&E Climate Zone | Heating | Cooling | Water Heating | Cooking | Process | Other |
| --- | --- | --- | --- | --- | --- | --- |
| Desert/Mountain | 35% | 2% | 32% | 26% | 2% | 3% |
| Valley | 32% | 5% | 20% | 10% | 29% | 4% |
| Coastal | 41% | 0% | 31% | 20% | 6% | 2% |
| Hill | 41% | 3% | 33% | 18% | 3% | 2% |
| Total | 39% | 2% | 30% | 19% | 7% | 3% |

Figure 1.   Map of PG&E climate zones.

(Source: PG&E 1999 Commercial Building Survey Report)

Step 5:  Allocate emissions by season.  The 1999 Commercial Building Survey Report provides the seasonal distribution of natural gas consumption for four PG&E climate zones (Table 11).

Table 11.   Quarterly natural gas consumption estimates for four

PG&E climate zones, reported in units of billion Btu.

(Source: PG&E 1999 Commercial Building Survey Report)

| PG&E Climate Zone | 1st Qtr | 2nd Qtr | 3rd Qtr | 4th Qtr | Total |
| --- | --- | --- | --- | --- | --- |
| Desert/Mountain | 2,337 | 1,232 | 1,103 | 2,167 | 6,839 |
| Valley | 2,371 | 1,419 | 1,384 | 2,231 | 7,405 |
| Coastal | 5,382 | 3,594 | 3,228 | 4,834 | 17,038 |
| Hill | 7,467 | 4,629 | 3,845 | 6,893 | 22,834 |
| Total | 17,557 | 10,874 | 9,560 | 16,125 | 54,116 |

#### Example Calculation

Below is an example calculation for commercial natural gas combustion, which has been carried out for Placer County.

Step 1:  For Placer County, the California Energy Commission estimated commercial natural gas consumption in 2000 to be 17 million therms, or 1.7 million MMBtu.  According to SACOG estimates, 2900 + 4043 people are employed in the mountain climate zone and 109,010 are employed in the valley climate zone, which also roughly corresponds to the split between the Mountain and Valley Air Districts.

Valley Usage = 1.7 MMBtu  109010  (109010 + 2900 + 4043) = 1.60 million MMBtu

Mountain Usage = 1.7 MMBtu  (2900 + 4943)  (109010 + 2900 + 4043)

= 0.10 million MMBtu

Step 2: Split commercial natural gas consumption by end use according to the percent distributions shown in Table 10.  The distribution of commercial natural gas combustion by area and end use for Placer County is shown below (units in MMBtu).

| PG&E Climate Zone | Heating | Cooling | Water Heating | Cooking | Process | Other |
| --- | --- | --- | --- | --- | --- | --- |
| Mountain | 32,764 | 4,907 | 21,096 | 11,159 | 27,877 | 4,011 |
| Valley | 513,298 | 76,883 | 330,501 | 174,821 | 436,734 | 62,846 |

Step 3: Apply assumptions about the type of combustion units employed for each end use, as follows.

End Use	Combustion Processes

Heating	Small boiler

Cooling	Turbine

Water heating	Small boiler

Cooking	Approximated as small boiler

Process heat or machinery	Small boiler (60%), turbine (20%), or engine (20%)

Other	Turbine (50%) or engine (50%)

The distribution of commercial natural gas combustion by process for Placer County is shown below (MMBtu).

| PG&E Climate Zone | Boilers | NG Engines | NG Turbines |
| --- | --- | --- | --- |
| Mountain | 81,744 | 7,581 | 12,488 |
| Valley | 1,280,660 | 118,770 | 195,653 |

Step 4:  Select emission factors from AP-42.  For a conservatively high estimate, choose the higher emission factor for each pollutant (Table 12).

Table 12.   AP-42 emission factors for natural gas combustion (lb/MMBtu).

|  | NOx | CO | SOx | VOC | TOC | CH4 | PM | PM10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Turbines | 0.32 | 0.082 | 0.0034 | 0.0021 | 0.011 | 0.0086 | 0.0066 | 0.0066 |
| IC Engines >90% load | 4.08 | 0.317 | 0.000588 | 0.118 | 1.47 | 1.25 | 9.99  10-3 | 9.99  10-3 |
| IC Engines <90% load | 0.847 | 0.557 | 0.000588 | 0.118 | 1.47 | 1.25 | 7.71  10-5 | 7.71  10-5 |
| Small boilers | 0.098 | 0.082 | 0.0006 | 0.0054 | 0.011 | 0.0023 | 0.0075 | 0.0075 |

Step 5:  Estimate emissions for each PG&E climate zone and end use.  Table 13 tabulates results for Placer County and Table 14 summarizes results for CCOS II participating counties.  As an example, VOC emissions are calculated as follows.

Table 13.   Annual emissions for commercial natural gas combustion for Placer County

(tons per year).

| Climate Zone | NOx | CO | SOx | VOC | TOC | CH4 | PM | PM10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mountain | 21.5 | 6.0 | 0.0 | 0.7 | 6.1 | 4.9 | 0.4 | 0.4 |
| Valley | 336.3 | 93.6 | 0.8 | 10.7 | 95.4 | 76.5 | 6.0 | 6.0 |

Table 14.   Annual emissions for commercial natural gas combustion (tons per year).

| County | NOx | CO | SOx | VOC | TOC | CH4 | PM | PM10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mendocino | 79.6 | 38.9 | 0.3 | 3.3 | 17.8 | 12.3 | 3.2 | 3.2 |
| Mariposa | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Nevada | 1.6 | 0.8 | 0.0 | 0.1 | 0.3 | 0.2 | 0.1 | 0.1 |
| Plumas | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Shasta | 105.8 | 52.8 | 0.4 | 4.3 | 22.5 | 15.2 | 4.4 | 4.4 |
| Sierra | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Tehama | 17.1 | 8.5 | 0.1 | 0.7 | 3.6 | 2.5 | 0.7 | 0.7 |
| Amador | 38.6 | 10.7 | 0.1 | 1.2 | 10.9 | 8.8 | 0.7 | 0.7 |
| Calaveras | 24.7 | 6.9 | 0.1 | 0.8 | 7.0 | 5.6 | 0.4 | 0.4 |
| El Dorado | 29.8 | 8.3 | 0.1 | 0.9 | 8.5 | 6.8 | 0.5 | 0.5 |
| Placer | 357.8 | 99.6 | 0.8 | 11.4 | 101.5 | 81.4 | 6.4 | 6.4 |
| Tuolumne | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Butte | 199.8 | 54.4 | 0.4 | 6.3 | 57.1 | 45.9 | 3.5 | 3.5 |
| Colusa | 178.7 | 48.7 | 0.4 | 5.6 | 51.0 | 41.1 | 3.1 | 3.1 |
| Glenn | 50.9 | 13.9 | 0.1 | 1.6 | 14.5 | 11.7 | 0.9 | 0.9 |
| Sacramento | 1780.5 | 485.0 | 3.9 | 56.1 | 508.5 | 409.2 | 30.9 | 30.9 |
| Solano | 356.3 | 97.1 | 0.8 | 11.2 | 101.7 | 81.9 | 6.2 | 6.2 |
| Stanislaus | 472.2 | 128.6 | 1.0 | 14.9 | 134.8 | 108.5 | 8.2 | 8.2 |
| Sutter | 119.8 | 32.6 | 0.3 | 3.8 | 34.2 | 27.5 | 2.1 | 2.1 |
| Yolo | 555.4 | 151.3 | 1.2 | 17.5 | 158.6 | 127.6 | 9.6 | 9.6 |
| Yuba | 67.7 | 18.4 | 0.1 | 2.1 | 19.3 | 15.6 | 1.2 | 1.2 |

Step 6:  Distribute emissions according to the seasonal variation in natural gas demand shown in Table 11.  Table 15 illustrates quarterly emissions estimates for Placer County.

Table 15.  Quarterly emissions from commercial natural gas combustion in Placer County (tpy).

| Climate Zone and Quarter | NOx | CO | SOx | VOC | TOC | CH4 | PM | PM10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mountain |  |  |  |  |  |  |  |  |
| 1st | 7.3 | 2.0 | 0.0 | 0.2 | 2.1 | 1.7 | 0.1 | 0.1 |
| 2nd | 3.9 | 1.1 | 0.0 | 0.1 | 1.1 | 0.9 | 0.1 | 0.1 |
| 3rd | 3.5 | 1.0 | 0.0 | 0.1 | 1.0 | 0.8 | 0.1 | 0.1 |
| 4th | 6.8 | 1.9 | 0.0 | 0.2 | 1.9 | 1.5 | 0.1 | 0.1 |
| Valley |  |  |  |  |  |  |  |  |
| 1st | 114.9 | 32.0 | 0.3 | 3.6 | 32.6 | 26.2 | 2.1 | 2.1 |
| 2nd | 60.6 | 16.9 | 0.1 | 1.9 | 17.2 | 13.8 | 1.1 | 1.1 |
| 3rd | 54.2 | 15.1 | 0.1 | 1.7 | 15.4 | 12.3 | 1.0 | 1.0 |
| 4th | 106.6 | 29.7 | 0.2 | 3.4 | 30.2 | 24.3 | 1.9 | 1.9 |

## References

EIA (2001a) State energy data report: consumption estimates.  Report no. DOE/EIA-0214(99) prepared by the Energy Information Administration, U.S. Dept. of Energy, Washington, D.C, May.  Available on the Internet from http://www.eia.doe.gov/emeu/sedr/contents.html.

EIA (2001b) Manufacturing Energy Consumption Survey, 1998.  Published by the Energy Information Administration, U.S. Dept. of Energy, Washington, D.C.  Available on the Internet from http://www.eia.doe.gov/emeu/mecs/.

EIA (2001c) Natural gas monthly, March 2001.  Monthly serial published by the Energy Information Administration, U.S. Dept. of Energy, Washington, D.C.  Available on the Internet from http://www.eia.doe.gov/oil_gas/natural_gas/data_publications/natural_gas_monthly/ngm_historical.html.

EIA (2000/2001) Petroleum marketing monthly.  Monthly report nos. DOE/EIA-0380(2000/04) through DOE/EIA-0380(2001/03) prepared by the Energy Information Administration, U.S. Dept. of Energy, Washington, D.C.  Available on the Internet from http://www.eia.doe.gov/oil_gas/petroleum/data_publications/petroleum_marketing_monthly/pmm_historical.html.

EIA (1998) Database file 20, consumption of natural gas, fuel oil, and district heat by end use.  From the commercial buildings energy consumption survey, 1995, prepared by the Energy Information Administration, U.S. Dept. of Energy, Washington, D.C.  Available on the Internet from http://www.eia.doe.gov/emeu/cbecs/microdat.html.

EIA (1997a) Manufacturing consumption of energy 1994.  Report no. DOE/EIA-0512(94) prepared by the Energy Information Administration, U.S. Dept. of Energy, Washington, D.C, May.  Available on the Internet from http://www.eia.doe.gov/emeu/mecs/mecs94/consumption/mecs4a.html.

EIA (1997b) How changing energy markets affect manufacturing.  In manufacturing consumption of energy – 1994, Chapter 4.  Report prepared by the Energy Information Administration, U.S. Dept. of Energy, Washington, D.C.

EPA (2000a) Stationary gas turbines.  In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), Supplement F (September 2000), Chapter 3.1.  Report prepared by Office of Air Quality Planning and Standards of the EPA, Research Triangle Park, NC.

EPA (2000b) Natural gas-fired reciprocating engines.  In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), Supplement F (September 2000), Chapter 3.2.  Report prepared by Office of Air Quality Planning and Standards of the EPA, Research Triangle Park, NC.

EPA (1999a) Fuel oil combustion.  In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), Supplement E (September 1999), Chapter 1.3.  Report prepared by Office of Air Quality Planning and Standards of the EPA, Research Triangle Park, NC.

EPA (1999b) Volatile organic compound/particulate matter speciation database management system (SPECIATE), version 3.1 (for microcomputers).  Prepared by the Office of Air Quality Planning and Standards, U.S. Environmental Protection Agency, Research Triangle Park, NC October.

EPA (1998) Natural gas combustion.  In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), Supplement D (August 1998), Chapter 1.4.  Report prepared by Office of Air Quality Planning and Standards of the EPA, Research Triangle Park, NC.

EPA (1996a) Gasoline and diesel industrial engines.  In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), Supplement B (November 1996), Chapter 3.3.  Report prepared by Office of Air Quality Planning and Standards of the EPA, Research Triangle Park, NC.

EPA (1996b) Liquefied petroleum gas combustion.  In compilation of air pollutant emission factors, vol. 1: stationary point and area emission units (AP-42), 5th ed. (January 1995), Supplement B (November 1996), Chapter 1.4.  Report prepared by Office of Air Quality Planning and Standards of the EPA, Research Triangle Park, NC.

US Census Bureau (2001) Censtats databases – county business patterns data.  Database maintained by the US Census Bureau, Washington, D.C., and accessed July 2001 at web site http://tier2.census.gov/dbappweb.htm.

Includes CA, OR, WA, AK, HI, MT, ID, WY, NV, UT, CO, AZ, NM

Pacific Gas and Electric forecast

Assume PM = PM10.